| Customer: | Mars Petcare US, Inc. | Machine Number: | M313029 |
| --- | --- | --- | --- |
| Contact: | Joseph Lofink | Proposal Number: | 13-000419A |
| Shipping Address: | 10000 Roberts Blvd. | Build-Up Number: | BU-13-01486 |
|  | Fort Smith, AR 72916 | Inquiry Number: | I1300363-01 |
| Purchase Order No.: | 1001518711 | Floor Plan Number: | 130560FPA 130674FPB |
| Machine Name: | Contour™ Series T-105 Tray Direct Load | Spec Origination Date: | 8/30/13 |
| Project Manager: | John Nori | Proposed FAT Date: | TBD |
| Phone: Fax: Email: | 320-762-9363 320-763-5754 john.nori@douglas-machine.com | Proposed Ship Date: | February 28, 2014 |

- PRODUCT DESCRIPTION:

Customer requested product capability and surge speed.

| No. | Product | Product Delivered | Direction of Travel A | Vertical B | Across Conveyor C | Requested Product Rate | Requested Surge Rate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 85 Gram Can | 1 | 2.60 | 1.44 | 2.60 | 1200 | 1,200 |

  - How is product received? Flood fed from customer’s upstream dynamic conveyor.

- PACK PATTERNS AND SPEEDS:

Machine product speed (may differ from requested product speed).

| No. | Product | Count | Pack Pattern |  |  | Direction of Travel A | Vertical B | Across Machine C | Guaranteed Tray Speed | Guaranteed Product Rate | Test Package |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A | B | C |  |  |  |  |  | A | B | C | D |
| 1 | 85 Gram Can - 1.31" Tray | 12 | 3 | 1 | 4 | 7.81 | 1.44 | 10.42 | 100 | 1200 | X |  |  |  |
| 1 | 85 Gram Can - 2.25" Tray with Layer Pad | 24 | 3 | 2 | 4 | 7.81 | 2.89 | 10.42 | 50 | 1200 | X |  |  |  |

Above chart is machine capability.  Transfer decks run multiple products in many cases, but additional decks/change parts and additional package formatting may need to be purchased to run all the sizes listed (see options section for testing).

Up to two (2) of the above package configurations are included in machine base price.  See options/pricing section for additional product configuration price.  The machine will arrive with only the pack configurations ordered.

Test Definitions:

Test Package A: This test package would include using the actual product to set-up and size the machine by doing a limited test run of product, manufacture any change parts, record the product recipe into the HMI and manuals, conduct a full Internal FAT until a satisfactory run is completed as listed in the machine spec, conduct a full Customer FAT until a satisfactory run is completed as listed in the machine spec.

Test Package B: This test package would include using the actual product to set-up and size the machine by doing a limited test run of product, manufacture any change parts, record the product recipe into the HMI and manuals, conduct a full Internal FAT until a satisfactory run is completed as listed in the machine spec.  NO CUSTOMER FAT RUNS

Package C: This package would include using product or product drawing to supply change parts (lane guide reference positioners, transfer deck, slip sheet, and preliminary HMI recipe into the HMI).  NO INTERNAL FAT TESTING OR CUSTOMER FAT TESTING.

Test Package D: This item would be for future products that either do not exist or are not run today.  This would be product dependent with Engineering hours built into design fit only.  There would be no samples, machine sizing, product testing, change parts or HMI recipe settings.

| - CORRUGATED DESCRIPTION: | Corrugated end slotted with 90° corners, 1-1/2” to 3” flap length. |
| --- | --- |
| - Glue pattern: | Two stripes per major flap. |
| - Direction of travel: | - Minimum 5” - Maximum 13” - Maximum flat tray blank dimension 21” |
| - Across machine: | - Minimum 9.25” - Maximum 22” - Maximum flat tray blank dimension 28” |
| - MACHINE ORIENTATION: |  |
| - Sequence of operation: | The blank will be removed by vacuum from the power fed magazine and delivered to the loading area. Loose product will be flood fed and standing upright, then arranged in the proper pattern and loaded onto the blank. After loading, the tray flaps will be glued with hot melt and sealed. The completed tray will be transferred to the customer conveyor. |
| - Infeed height: | Standard: 53-1/4” with 4-1/2” frame to floor clearance. Adjustability: Minimum 51-1/4” to 58”. |
| - Discharge height: | Standard: 53-1/4” with 4-1/2” frame to floor clearance. Adjustability: Minimum 51-1/4” to 58”. |
| - SHIPPING SIZE REQUIREMENTS: | Van: Maximum size 95” wide by 100” high x lengths determined by machine design. Container: Maximum size 92” wide by 101.5” high x lengths determined by machine design. |
| - EQUIPMENT SPECIFICATIONS: | This machine is designed with high quality components for standardization. Any changes other than those listed in options may not be available on this model, or will have an effect on price and delivery. |
| - OPERATING CONDITIONS: |  |
| - Ambient temperature: | 50°- 100° F. |
| - Humidity: | Recommended 75% maximum. |
| - UTILITY REQUIREMENT: |  |
| - Machine supply voltage: | 480V-3ph-Grounded WYE 60Hz. |
| - Air requirements: | 90 psi, clean, dry. |

| - MECHANICAL FEATURES: |  |
| --- | --- |
| - Requirements for customer product delivery: | Customer to provide product flood fed over the center of our conveyor tail shaft. We may require an upstream speed reference and control of upstream conveyors. |
| - Product infeed: | Dual recirculation style infeed. |
| - Product accumulation: | Product accumulation and lane arrangement for pack pattern. |
| - Product metering: | Overhead servo fixed pin metering. |
| - Blank magazine/setup: | Ten-foot (10’) blank magazine with rotary pick capable of holding 960 corrugated blanks. |
| - Warped corrugated vacuum assist: | Warped corrugated vacuum assist box with two turbines NEMA 12 electrical. (Minimum corrugate or product across machine dimension 9.25”). |
| - Blank delivery and load: | Bottom up blank delivery with product sweep direct load. Customer selected option 9: Provide 90 degree blank delivery with location for two coders to mount. Blank travel rate at 100 trays per minute is 225 feet per minute. The coder manufacturer must guarantee coder capable of applying customer code at that rate. The customer will supply Markem code heads. Douglas will supply the following: brackets for mechanical mounting of the code heads, positioned to code the long side of the tray. The brackets will have as much adjustment as practical. |
| - Vacuum system: | Busch oil-less. |
| - Glue system: | Nordson ProBlue™ Series 10 480 volt hot melt applicator with two (2) Solidblue (H201T) guns with 90 degree nozzles providing two stripes per flap. Saturn filter with 100 mesh provided before the guns. Separate 480V NEMA 12/4X glue drop with separate power disconnect. Customer selected option 8: Nordson fulfill system - Integrated, automatic fill system maintains adhesive levels in ProBlue melters with additional 132 lb storage capacity. |
| - Hot melt adhesive: | There are many factors considered in the selection of an adhesive for sealing cases and cartons. Factors include, but are not limited to; mechanical, substrate, plant environment, end use, as well as the storage conditions of glue, packaging materials, and finished product. The customer is responsible to ensure any adhesive selected meets all of the machine and end use requirements. |
| - Machine frame: | Continuous weld tubular cold-rolled steel. |
| - Leg style: | Stainless steel threaded rod legs with fixed base (accommodates up to 9º slope). |
| - Fabricated components: | Powder-coated, nickel plated and QPQ cold rolled steel, clear and hard anodized aluminum, plastic or stainless steel, dependent upon function with continuous welding where possible. |
| - Fasteners: | Plated carbon steel and stainless steel. Set screws will be carbon steel. |
| - Linear rails and guides: | APC plated guides and rails. |
| - Bearings: | Carbon steel bearings. |
| - Brand of servo motors: | Allen-Bradley MPL servo motors (IP65). |
| - Brand of AC motors: | Lenze High Efficiency (IE2) wash down IEC motor. |
| - Brand of DC motors: | Magazine drives – 90 volt standard duty. |
| - Gear motors: | Lenze High Efficiency motors w/Lenze High Efficiency helical bevel gear 90 degree sealed multi-position gear box. |
| - Gear boxes: | Lenze High Efficiency (painted aluminum), Alpha (painted aluminum), Hub City (painted). |
| - Adjustment indicators: | Scales in inches for slotted adjustments and digital indicators for handwheels. Digital indicators have polymer housings and stainless steel collars. |
| - Chains: | Renold Jeffrey Hydro service drive and adjustment chains. |
| - Hardened sprockets: | All roller chain sprockets, other than sprockets used for adjustments. |
| - Perimeter machine guards: | Gull wing center pivoting. Wide opening for easy clear access for operators and maintenance. |
| - Guard frame: | Anodized aluminum. |
| - Doors and panels: | Clear ¼” polycarbonate in aluminum frame. |
| - Safety: | Guard doors with handles and interlocks are installed where machine operators may need access to the machine. All other guard doors will require a tool to open and/or remove. |
| - Lubrication system: |  |
| - Grease bank system: | The majority of bearings are non-relube minimum maintenance bearings. For bearings and bushings requiring lubrication, the lines are stainless steel and flexible plastic. Linear bearings and rod ends will have their standard fittings. |
| - Type of grease in system: | Food grade. |
| - Type of oil in system: | Food grade. Food grade oil not available in sealed gear heads, phasing gearboxes, indexers and servo gearboxes. Synthetic oil/grease is used in servo gearboxes. |
| - Powder-coat: | Dupont 616 gray metallic epoxy. Purchased components are supplied with manufacturers’ standard finish. |
| - Overload detection: | Stops the machine when actuated and displays the location on the HMI. |
| - Additional components: | - Aurora steel insert rod ends - Dodge/Hub City/SKF ball bearings - Hub City, Lenze, Alpha gearbox - Mayr overload clutch - McGill or Smith cam followers - Nexen clutch/brake unit - THK linear slide rails |
| - Mechanical miscellaneous: | - None - Customer selected option 5: Servo overhead front containment bar for PET or tippy product. - Customer selected option 23: Provide riveted phenolic tags in most places in lieu of glued tags. |

| - ELECTRICAL FEATURES: |  |
| --- | --- |
| - Control voltage: | 24V DC. |
| - NEMA rating: | NEMA 4 wiring and components. NEMA 4 cabinets (main cabinet is NEMA 12). Cabinet cooling also de-rates NEMA 4 to NEMA 12. Cabinets are powder coated . |
| - Electrical cabinets: | Powder-coated Dupont 616 gray metallic epoxy electrical cabinets. Customer selected option 14: Cabinet light for main/PLC cabinet with door switch. |
| - Electrical cabinet cooling: | Two (2) 4000 BTU NEMA 12 conditioners. |
| - Conduit: | Galvanized conduit, cable trays, and powder coated wireway system. |
| - Douglas wiring separation: | Wires are separated by metal divider wireway, cable tray or separate conduit. 1. Low voltage: 24V, Ethernet, and analog signals. 2. High voltage: 480V, 120V, 90V DC. Shielded cables are used for VFD and servo cabling. |
| - Programmable logic controller: | Allen-Bradley ControlLogix L72 processor. |
| - I/O module: | AB distributed I/O system. |
| - Communication port: | Grace communication port and receptacle mounted to the outside of the main electrical enclosure. |
| - Servo system: | Allen-Bradley Kinetix. |
| - Operator Interface (HMI): | Allen-Bradley PanelView Plus1000 color display with machine control and annunciation screens. Customer selected option 20: Onboard documentation system with HMI in place of PanelView Plus 1000 HMI. Includes a custom service manual digitally formatted for optimal clarity and display, intuitive navigation features and a PanelView Plus 1500 HMI (15 inch display). |
| - HMI analog magazine control: | Analog magazine speed control. |
| - HMI analog glue control: | Glue run-up control for glue conservation. |
| - Variable frequency drive: | Allen-Bradley PowerFlex 40P. |
| - Machine timing: | Servo controlled. |
| - Safety circuits: | Douglas Machine engineers its equipment in accordance with EN 954-1 or EN ISO 13849 CAT 1 for pneumatic systems, and EN 954-1 or EN ISO 13849 CAT 3 for electrical safety circuits, including but not limited to, the following electrical components: |
| - Safety relays: | Allen-Bradley 440R. |
| - Interlock switches: | Allen-Bradley 440N. |
| - Emergency stops: | Allen-Bradley 800H (2 standard, 1 with HMI and 1 remote). |
| - Main disconnect switch: | AB or Ferraz Shawmut lockable flange mount with fuses. |
| - Down product: | Detection on infeed and indication. |
| - Missing container count eyes: | Missing container count eyes - up to eight (8) lanes checked within the machine for cycle stop capability. |
| - Open flap: | Detection and indication. |
| - Low magazine: | Low level detection and countdown to empty. |
| - Downstream surge control: | Dwells machine if cases are detected at discharge. |
| - Line communications: | Dedicated Ethernet card provided |
| - UL panels: | UL approved electrical panels. |
| - Warning horn: | Delayed start with horn. |
| - Stack lighting: | Allen-Bradley 3-color lights. |
| - Individual guard door annunciation: | Detection and indication of individual guard door open. |
| - Additional components: | - Allen-Bradley pushbuttons, and motor control - Allen-Bradley photo eye controls - Bussmann fuses and fuse blocks - Hoffman electrical enclosures - Turck proximity switches - Micro Switch Limit switches - Square D Limit switches - AB spring clamp terminals - Idec control relays - Hevi-Duty transformers |
| - Electrical miscellaneous: | - None - Customer selected option 13: Individual motor disconnects - Hubble (HBLDS3) NEMA 4X motor disconnect (four disconnects). - Customer selected option 15: PLC power conditioner - Islatrol line filter. - Customer selected option 16: Sola Isolation Transformer. - Customer selected option 17: Provide fused inputs. |
| - CLEANING: | Douglas Machine Shrink-Wrap Systems are not designed to be operated in washdown environments. Under no circumstances should water, or any other liquid, be directed at any part of the wrapper or tunnel. Product spillage should be removed by wiping. |
| - Machine structure: | Wipe off machine components using a dry rag. Use dry compressed air, maximum of 40 PSI, to remove accumulations such as corrugated dust. Do not direct air pressure to sensitive areas of the machine such as the inside of electrical cabinets, electrical devices, bearing seals, hot melt applicator, etc. |
| - Lexan guard panels: | Clean with soft, damp cloth. Abrasive pads should not be used. |
| - Clarification: | Options such as NEMA 4X enclosures, stainless steel fasteners, etc., are intended for aid in resistance to corrosion. However, commercially supplied components such as bearings, gearboxes, reducers, valves, torque limiters, clutches, hot melt applicator, etc., are typically installed with their manufacturer’s standard supplied finish (normally painted), and are not necessarily waterproof or corrosion resistant. Detergents, high-pressure water, and/or caustic solutions (alkaline/acidic), applied to this machine, may cause component malfunction and/or corrosion. |

| - LANGUAGE: |  |
| --- | --- |
| - Manuals: | English. |
| - Warning signs: | English. |
| - Human Machine Interface (HMI): | English. |
| - SPARE PARTS: | Not included. Offered as an option. Customer selected option 24: Spare parts kit - The final recommended spare parts list is compiled after the machine is completed (estimated price). |
| - MACHINE EFFICIENCY: | 97% Consult Douglas Machine Inc. for specifics. Introduction of dry conveyor lubricant systems at the entrance to Douglas infeed conveyors is not recommended. Excessive carryover of conveyor lubricant from upstream must be controlled or efficiencies will not be achieved. |
| - FAT AT DOUGLAS MACHINE: | Acceptance testing of the machine with customer-supplied materials will consist of two 5-minute runs per product size and pack pattern. High speed/volume machines may require accumulative runs to achieve the 5 minutes. |
| - FAT AT CUSTOMER PLANT: | Acceptance testing of the machine will consist of 2 consecutive 8-hour runs. Douglas Machine personnel will be available to support this test at our current technical support rates. See attached rate sheet. Consult Douglas Machine Inc. for specifics. |

| Machine Items: |  |
| --- | --- |
| Included pack configurations: | Two (2) pack configurations include: change parts, programming, documentation and factory testing. |

| Machine Options |  |
| --- | --- |
| - Overhead front containment: | Servo overhead front containment bar for PET or tippy product. |
| - Nordson fulfill system: | Integrated, automatic fill system maintains adhesive levels in ProBlue melters with additional 132 lb storage capacity. |
| - 90 degree blank delivery: | Provide 90 degree blank delivery with location for two coders to mount. Blank travel rate at 100 trays per minute is 225 feet per minute. The coder manufacturer must guarantee coder capable of applying customer code at that rate. The customer will supply Markem code heads. Douglas will supply the following; brackets for mechanical mounting of the code heads, positioned to code the long side of the tray. The brackets will have as much adjustment as practical. |
| Electrical Options |  |
| - Individual motor disconnect: | Hubble (HBLDS3) NEMA 4X motor disconnect (four disconnects). |
| - Cabinet lighting: | Cabinet light for main/PLC cabinet with door switch. |
| - PLC power conditioner: | Islatrol line filter. |
| - Isolation Transformer: | Sola Isolation Transformer. |
| - Fused Inputs: | Provide fused inputs. |
| - Onboard documentation system with HMI: | Onboard documentation system with HMI in place of PanelView Plus 1000 HMI. Includes a custom service manual digitally formatted for optimal clarity and display, intuitive navigation features and a PanelView Plus 1500 HMI (15 inch display). |
| Mechanical Options |  |
| - Riveted phenolic tags: | Provide riveted phenolic tags in most places in lieu of glued tags. |
| Other Options |  |
| - Spare parts kit: | The final recommended spare parts list is compiled after the machine is completed (estimated price). |

Customer product samples provided? 	No

Customer product drawing provided? 	Yes

Customer case blank sample provided? 	No

Customer case blank drawing provided? 	Yes

Douglas Machine reference drawing?

Customer specifications provided? 	Yes

List customer specifications that apply.

List all exceptions and clarifications to customer specifications.

Mars Petcare - (CS00986) Can Pkg'g Sys Specs_Final w Pictures Rev 1 Exceptions

Page: 2 Author: RodT

- Can Packaging System.4: Clarification: Blank to be corrugate.

Page: 6 Author: RodT

- 12 Count Format - 85g Aluminum Cans.4: Exception: Reject mechanism not provided.

- 12 Count Format - 85g Aluminum Cans.6: Exception: Not providing.

- 12 Count Format - 85g Aluminum Cans.7: Exception: Not providing.

Page: 9 Author: RodT

- 24 Count Format - 85g Aluminum Cans.5: Exception: Reject mechanism not provided.

- 24 Count Format - 85g Aluminum Cans.7: Exception: Not providing.

- 24 Count Format - 85g Aluminum Cans.8: Exception: Not providing.

- Machine Features.2: Exception: Nordson is standard.

Page: 10 Author: RodT

- Operational and Maintenance Considerations.2: Clarification: Douglas Machine Shrink-Wrap Systems are not designed to be operated in washdown environments. Under no circumstances should water, or any other liquid, be directed at any part of the wrapper or tunnel. Product spillage should be removed by wiping.

Page: 12 Author: RodT

- Mechanical Considerations.6: Exception: Stainless steel threaded rod with stainless steel self-leveling bolt base as standard. Provide option pricing if required.

Mars Petcare - (CS00987) Mars Inc. Machine Guarding Stds

Page: 6 Author: RodT

- Risk Assessment: Clarification: Douglas Machine uses Design Safe 5 software to complete the risk assessment.

Mars Petcare - (CS00988) Cntrl Sys Stds - Fort Smith v1.7 April 2011

Page: 6 Author: justinp

- 6. Exception: Douglas Standard Compliance Statement

Page: 8 Author: justinp

- 2.1.5 Exception: "Mars Petcare US New Petfood Site Ethernet Guidelines for PLC Systems" was not provided.

Page: 9 Author: justinp

- 2.3.1 Exception: 0-10V and +-10V signals may be used.

Page: 12 Author: justinp

- 4.1.5 Option: Separate control power for PLC and Servo Drives.

- 4.1.5 Option: Customer supplied PLC control power.

Page: 13 Author: justinp

- 4.3.5 Clarification: Machine will be NEMA 12.

- 4.3.6 Clarification: Machine will be NEMA 12.

- 4.3.7 Exception: Main Enclosure will include a lockable flange style disconnect. Other machine enclosures that contain 480VAC will not have disconnects. These enclosures can have lockable handle installed.

- 4.3.8 Exception: Douglas standard design will be used. No disconnect on 120VAC in enclosures.

- 4.3.9 Exception: Douglas standard design will be used. Main Enclosure contains ground bar.

- 4.3.9 Option: Lighting in all enclosures.

- 4.3.11 Exception: Douglas standard panel design will be used, high rise DIN rail may not be used for all terminals.

- 4.3.11 Exception: 4 conductor terminals will be used and up to 4 wires may be installed in each terminal.

- 4.3.12 Exception: Din Rail may be made of aluminum.

Page: 14 Author: justinp

- 5.1.2 Exception: PowerFlex 525 drives will be used, they do not use the 20-COMM-E adapter, but are Ethernet/IP drives.

- 5.1.4 Exception: Douglas standard programming will be used.

- 5.1.5 Clarification: Safe-Off option boards are built in to PowerFlex 525 drives.

Page: 15 Author: justinp

- 5.1.7 Exception: Douglas standard wiring will be used.

Page: 16 Author: justinp

- 5.7.1 Exception: Some sensors will be wired directly to Point I/O terminal. I/O in the PLC enclosure will be wired to terminal strips.

- 5.7.1 Douglas standard panel design will be used, high rise DIN rail may not be used for all terminals.

Page: 16 Author: justinp

- 5.7.2 Option: Fused Inputs

- 5.7.3 Exception: Allen Bradley Point I/O will be used, not Flex I/O.

- 5.7.4 Exception: Outputs will be electronically fused.

- 5.7.7 Exception: 4 conductor terminals will be used and up to 4 wires may be installed in each terminal.

- 5.8.1 Exception: 18 AWG may be used in some cases.

Page: 17 Author: justinp

- 5.8.4 Exception: Douglas standard wiring will be used.

- 5.9.4 Exception: "Mars Petcare US Machinery Safety.doc" was not provided.

Page: 18 Author: justinp

- 6.3.2 Exception: "Mars Petcare US New Wet Petfood Site Programming Standard" was not provided. Douglas standard programming will be used.

Page: 19 Author: justinp

- 7 Clarification: Rockwell Factory Talk View ME will be used for machine control.

- 7.1.4 Exception: "Mars Petcare US New Wet Petfood Site Programming Standard" was not provided. Douglas standard programming will be used.

Page: 21 Author: justinp

- D. Clarification: Does this section apply to machine OEM or is this for Plant Wiring/Contractors?

Page: 22 Author: justinp Subject:

- 3.1.1 Exception: Douglas standard compliance statement.

- 3.1.5 Exception: The listed Mars Petcare US Standard

- Operating Procedures and Specifications documents were not provided.

Page: 28 Author: justinp

- 7.6.6 Exception: Attenuation pads will not be provided.

Page: 30 Author: justinp

- 9.2.1 Exception: 18 AWG may be used in some cases.

- 9.2.2 Exception: Wire color coding will follow NFPA 79.

- 9.2.6 Exception: MTW will be used for panel wiring.

Page: 31 Author: justinp

- 10 Exception: Wire way and cable tray will be used.

Page: 34 Author: justinp

- 11.6.8 Exception: Enclosure drains will not be provided.

Page: 35 Author: justinp

- 11.8.1 Exception: Douglas standard legend plate sizes will be used.

Page: 37 Author: justinp

- 12.4.7 Option: Motor disconnects are an option.

Page: 38 Author: justinp

- 12.5.4 Option: Motor wiring phase marking is an option.

Page: 39 Author: justinp

- 12.7 Exception: Wire way or cable tray may be used.

Page: 46 Author: justinp

- 12.14.2 Exception: Standard Lipped style junction boxes will be used.

- 12.14.1 Exception: Boxes may be made of 16 gauge steel.

Page: 47 Author: justinp

- 12.15.3 Exception: Wireway may contain holes to allow cables to enter.

- 12.15.3 Exception: Wireway may be made of 16 gauge steel.

Page: 48 Author: justinp

- 12.16.7 Exception: Cables will not be in single layer.

Page: 49 Author: justinp

- 12.16.8 Exception: Douglas standard wiring separations will be used. Low voltage: 24V, Ethernet, and analog signals. High voltage: 480V, 120V, 90V DC. Shielded cables are used for VFD and servo cabling.

Page: 57 Author: justinp

- 18.1.5 Exception: Douglas standard wiring separations will be used. Low voltage: 24V, Ethernet, and analog signals. High voltage: 480V, 120V, 90V DC. Shielded cables are used for VFD and servo cabling.

Page: 64 Author: justinp

- 21.3.3 Exception: 480V fuses will be LPJ and DFJ type.

Page: 70 Author: justinp

- 2.2 Exception: Douglas standard wire numbering will be used. Examples can be provided.

Page: 71 Author: justinp

- 2.3 Exception: Black will be used for Phases. Color Tape can be added at each end of wire to indicate Phase.

- 2.3 Exception: CAT 5E will be used for all communication and it will be Green.

Page: 72 Author: justinp

- 3.1 Exception: 1756-L7x series will be used.

Page: 73 Author: justinp

- ControlLogix Permissible Components: ControlLogix CPU - Exception: 1756-L7x

- ControlLogix Permissible Components: Power Supply 120VAC -Exception: 1756-PA72

- ControlLogix Permissible Components: Exception: Other Components that will be used:

  - 1756-OB16E

  - 1756-IB16

Page: 74 Author: justinp

- 3.2 Exception: Other Components that will be used:

  - 1734-IB4

  - 1734-OB4E

  - 1734-IE2V

  - 1734-OE2C

  - 1734-IE2C

Page: 75 Author: justinp

- 3.3 Exception: Ethernet Switches used will be N-Tron 708TX, 108TX, or 105TX

Page: 76 Author: justinp

- 3.5 Option: PanelView Plus 1500. If taken, part number will be: 2711P-T15C4D8.  If option not taken PanelView Plus 1000 will be provided. Part number: 2711P-T10C4D8.

- 3.6 Pushbuttons, Selectr Switches - Exception: 800HC will be used.

- 3.6 Enunciator - Exception: 855E will be used.

Page: 78 Author: justinp

- 3.7 Guard Door - Interlock Switches (non-contact) - Exception: 440N-Z21W1PH

Page: 79 Author: justinp

- 3.7 At-Motor Disconnects; Local Disconnect Switches - Option: Motor Disconnects.

- If selected will be Hubble part number:  HBLDS3

- 3.7 Disconnects - Exception: AB 194R series will be used.

Page: 80 Author: justinp

- 3.8 Photo-electric - Exception: Allen Bradley and Sick Photo-Electrics will be used.

- 3.9 Fuse Holders - Exception: Bussmann JTN Series

Page: 81 Author: justinp

- 3.10 Exception: Idec will be used

- 3.11 Exception: 104S-C will be used.

Page: 82 Author: justinp

- 3.12 Exception: Sola/Hevi Duty HS series.

- 3.12 Exception: 1606-XLS series

- 3.14 Option: PowerFlex 70 Drives.  PowerFlex 525 is standard.

Page: 83 Author: justinp

- 3.17 Exception: Motors will be Lenze.

Page: 84 Author: justinp

- 3.18 Exception: 1492-L series will be used.

- 3.18 Exception: E-Rail DIN Rail will be used.

- 3.18 Exception: Ferraz Shawmut: FSPD3A

- 3.18 Exception: Bussmann JTN Series

Page: 86 Author: justinp

- 3.20 Exception: NEMA 12 Enclosures will be used.

- 3.21 Exception: Numatics VS22-03

- 3.21 Exception: MAC valves.

Page: 90 Author: justinp

- 1 Exception: Douglas standard schematics will be used.

- 2 Exception: Douglas standard enclosure drawings will be used.

| Documentation: | Douglas' Technical Publications Support provides informative and easy to understand Operator and Maintenance machine documentation including: - Two operator manuals (provided at machine shipment). - One set of vendor supplied OEM bulletins (provided at machine shipment). - Two service manuals (provided 10 to 14 days after machine shipment). - One CD containing (provided 10 to 14 days after machine shipment): - Operator Manual (.pdf format) - Service Manual (.pdf format) - Bill of Materials (.xls format) - Recommended Spare Parts List (.xls format) - Original Changeover settings (.xls format) - Original Recipe Variable Settings (,xls format) - Assembly drawings (.pdf format) - Electrical drawings (.pdf and .dxf format) - Electrical program (PLC program) - Operator screens (.pdf and HMI program) - Two sets of job aids (printed and electronic - .pdf only) (provided 10 to 14 days after machine shipment: - Changeover Settings Table - Changeover Quick Reference Guide Handbook - Troubleshooting Quick Reference Guide Handbook RISK ASSESSMENT: Douglas Machine has committed itself to providing its customers with the highest quality machines in the area of safety through design. As a result of our comprehensive efforts to improve the quality as well as the safety of our machines, our risk assessments have become much more descriptive of our designs and therefore are considered to be the intellectual property of Douglas Machine Inc. The risk assessment now contains descriptions of areas that are proprietary to Douglas Machine and must remain that way. Because of this, Douglas Machine risk assessment documents shall not be duplicated, copied, reproduced or distributed in any manner outside of Douglas Machine, nor shall these documents leave the premises of Douglas Machine Inc. This document should be reviewed as part of the design reviews between our two companies, or per request at the Douglas Machine Inc. facility. |
| --- | --- |
| Documentation Shipping Address: | Mars Petcare US, Inc. Attn: Joseph Lofink 10000 Roberts Blvd. Fort Smith, AR 72916 |

| TRAINING DETAILS: |  |
| --- | --- |

| InSTALLATION DETAILS: |  |
| --- | --- |

| SPARE PARTS DETAILS: |  |
| --- | --- |

Customer selected option 24: Spare parts kit - The final recommended spare parts list is compiled after the machine is completed (estimated price).

All new packaging equipment manufactured and supplied by Douglas Machine Inc. is warrantied to be free from defects in materials and workmanship for the period of time specified in this statement. The warranty period commences from the date the equipment is shipped from our facility.

- Base Frame	Lifetime

- Douglas Standard Components	3 years

- Buyer Specified Components	1 year

The following limitations and restrictions apply to this warranty:

- The base frame limited lifetime warranty applies to structural and welding failures such as breaking, cracking or bending.

- Douglas Standard Components include all non-wear parts that are fabricated or purchased by Douglas Machine and incorporated into the supplied equipment. However, select standard components are warrantied for less than three (3) years depending upon the original manufacturer’s warranty. Consult with Douglas Machine for warranties on standard components.

- Buyer Specified Components include all non-wear parts that differ from Douglas’ Standard Components and are incorporated into the supplied equipment per Buyer’s specification requirements. However, Buyer Specified components are warrantied for one (1) to five (5) years depending upon the original manufacturer’s warranty.

- Most wear components shall not be warrantied. However, select wear components are warrantied for one (1) to five (5) years depending upon the original manufacturer’s warranty. A warranty claim against a wear part may be made if it is determined the part failed due to a factory defect. If it is determined the part failed due to a factory defect, the claim will follow the normal returns process. Consult with Douglas Machine for warranties on wear components.

- Wear and tear on the equipment that is considered standard in the industry shall not be warrantied.

- Equipment that is purchased by Douglas Machine and incorporated into the supplied equipment shall be subject to the original manufacturer’s standard warranty. Such equipment includes, but is not limited to, conveyors, accumulators, inspection systems and coders.

- Unless specified otherwise, frame and component finishes shall not be warrantied if the equipment is installed in a wet environment, or exposed to harsh cleaners or high-pressure wash downs.

- Equipment that has been misused, poorly maintained, neglected, or improperly moved, repaired or altered by anyone other than Douglas Machine or its authorized agents shall not be warrantied. Corrosion on components not manufactured by Douglas Machine shall be warrantied as per the original manufacturer’s standard warranty.

- Douglas Machine shall not be responsible for any removal, handling, or shipping charges (including applicable duties, taxes and inspection fees) that the Buyer incurs to return a component for warranty evaluation.

- Douglas Machine shall be responsible for the repair or replacement of components that are, in its sole determination, defective and covered by warranty. This responsibility includes handling charges, non-expedited shipping charges to return a component to the Buyer, and all applicable duties, taxes and inspection fees.

- Douglas Machine shall not be responsible for any labor costs to reinstall components or for line downtime during repair.

- This warranty is for the sole benefit of the original Buyer and is not transferable.

- This warranty is in lieu of all other warranties, expressed or implied, or merchantability, or fitness for a particular purpose, performance or otherwise.

- This warranty applies to new packaging machinery only. Modified or reconditioned equipment is not warrantied.

Douglas standard components include all non-wear parts that are fabricated or purchased by Douglas Machine and incorporated into the supplied equipment.

Most wear components shall not be warrantied. However, select wear components are warrantied for one (1) to five (5) years depending upon the original manufacturer’s warranty. A warranty claim against a wear part may be made if it is determined the part failed due to a factory defect. If it is determined the part failed due to a factory defect, the claim will follow the normal returns process.

See list below for a description and warranty period of Douglas standard components and wear and non-wear components

| 3-YEAR WARRANTY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Manufacturer | Item(s) |  | Manufacturer | Item(s) |

| Allen Bradley | Safety Disconnect, AC and Servo Drives, Servo Line Filter, Servo Cables, Absolute and Incremental Encoders, HMI, Manual Motor Control, Motors, Photo Controls (photoeyes), Programmable Logic Controller (Processor, I/O, rack), Switches (pushbuttons/pilot lights 800H style) |
| --- | --- |
| Balluff | Switch (proximity) |
| Banner | Photo controls (photoeyes) |
| Bimba | Cylinders (pneumatic) |
| Boston | Gears |
| Elau | Servo Drives |
| Fenner | Tensioners |
| Gates | Pulleys |
| Igus | Energy Chain |
| Linn | Gears, Sprockets |
| Murr | Line Filter (servo) and On-Machine I/O |
| Onvio | Pulleys and Gear Boxes (seals : 2 years) |
| Ringfeder | Adaptors |
| Ruland | Coupling (rigid) |
| Siemens | AC Drives, HMI |
| THK | Linear Rails and Guides |
| TRD | Cylinders (pneumatic) |
| Turck | Switch (proximity) |
| Wittenstein | Gear Boxes (seals: 2 years) |
| Watlow | Temperature Control (power and temperature) |

| 2-YEAR WARRANTY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Manufacturer | Item(s) |  | Manufacturer | Item(s) |

| Busch | Vacuum Pump |
| --- | --- |
| Elau | Motors (servo), Programmable Automation Controller (processor I/O rack) |
| Interroll | Drum Motors |
| Kenall | Light (cutter assembly) |
| Lenze | Gear Motors and Gear Boxes |
| Mayr | Torque Limiter (axial friction, single revolution, rotary) |
| SMC | Cylinders (pneumatic) |
| Sola | Power Supply UPS |

| 1-YEAR WARRANTY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Manufacturer | Item(s) |  | Manufacturer | Item(s) |

| Air Vac | Vacuum (Venturi) |
| --- | --- |
| American Fan | Impeller |
| Baldor | AC/DC Motors |
| Bando | Pulleys |
| Beckett Air | Impeller |
| Breco | Pulleys |
| Browning | Gears, Pulleys, Coupling, Tensioners |
| Camco/Ferguson | Indexers |
| Dart | DC Drives |
| Dayton | Cooling Fan |
| Duff Norton | Acuator |
| Edco | Vacuum (Venturi) |
| Fairchild | Gear Box |
| Ferraz Shawmut | Switch (disconnect) |
| GAM (Jakob) | Coupling (servo) |
| Goldenrod | Airshafts |
| Hevi-Duty | Transformer (power conditioning) |
| Hoffman | Enclosure (air conditioners, main operator, junction, fan, light fixture) |
| Hubbell | Disconnect (safety) |
| Hub City | Gear Box, Sealed Lubeable Ball Bearing Inserts |
| Industrial Fiber | Fibers (SERCO) |
| IPTICI | Bearings (mounted washdown only), Sealed Lubeable Ball Bearing Inserts |
| Leeson | AC/DC Motors |
| Light Exchange | Light Fixture (enclosure) |
| Lincoln | Lube (pump) |
| Lovejoy | Coupling (flexible), Motor Mounts, Tensioners |
| Islatrol | Line Filter (PLC and servo) |
| MAC | Valves (pneumatic) |
| Marathon | DC Motors |
| Martin | Gears, Sprockets |
| Maska | Pulleys, Taper Lock Bushings |
| Maurey | Pulleys |
| McMaster Carr | Casters |
| Micro-Switch | Switch (limit) |
| Midwest Motion Products | Acutator |
| Morse | Coupling (flexible), Torque Limitor (friction) |
| Nexen | Brake (pneumatic), Clutch (pneumatic), Disk Brake |
| Nordson | Glue System (guns, hoses, tank) |
| Numatic | Cylinders (pneumatic), Filter Housing (pneumatic), Regulator (pneumatic) |
| SMC | Switch (pressure and vacuum) |
| PHD | Guided Cylinders (pneumatic) |
| Phoenix Contact | Switch (Ethernet) |
| Piab | Vacuum (Venturi) |
| Robohand | Cylinders (pneumatic slides) |
| Schaffner | Line Filter (PLC and servo) |
| Seal Master | Sealed Lubeable Ball Bearing Inserts |
| Simco | Power Supply (static bar) |
| Sixnet | Modem (telephone) |
| Square D | Switch (limit) |
| SST | Sprockets |
| Stober | Gear Box |
| Tidland | Airshafts (shrink film) |
| Warner | Electric Clutch, Clutch (ratcheting) |
| Zeromax | Coupling (flexible, servo) |

WEAR COMPONENTS NOT UNDER WARRANTY (Including, but not limited to, the components listed below.)

Air Cylinders

Air Filters

Batteries

Bearings – except washdown (1 year)

Belts – Conveyor

Belts – Drive

Belts – General

Belts – Servo Drive

Brake Pads

Brushes

Bulbs (light)

Bushings

Cabling Flexible

Cabling – General

Cam Follower

Chain and Rails

Control Relays

Couplers

Crush Rollers

Fasteners

Fuses / Breakers

Gas Springs / Gas Shocks

Guard Door Switches

Heaters

Hoses

Light Emitting Diodes

Linear Bearings, Shafts, Rails

Mat-Top Chain

Nice Bearings

Plastic

Rod Ends

Roller Chain

Rollers

Safety Relays

Seals

Solenoid Valves

Springs

Sprockets

Static Bars

Table-Top Chain

Tunnel Heaters

Vacuum Filters

Vacuum Cups

Vulcanizing

Wrap-Spring Clutch

Wrapper Knives

If any component purchased from Douglas Machine does not work, or if the incorrect component is shipped, simply return it to us for full credit or replacement within 30 days of purchase. Please note that if the incorrect component was shipped, Douglas Machine must be contacted within 30 days of the purchase or the return cannot be processed. Upon notification, we will immediately correct the situation.

We may, at our option, assess a 15% restocking charge if the component was incorrectly specified and/or damaged beyond reasonable repair. Incorrectly specified components that have been misused and/or show signs of wear will not be accepted for return.

#### Processing Your Return

When returning a component for warranty evaluation or repair, these simple steps will help us expedite your request.

- Contact our Technical Support Department and request a Return Goods Authorization (RGA) number;

- When preparing the component for return shipment, please include the following information: (a) the machine serial number, (b) a detailed description of the problem you are experiencing and, (c) a date code sticker, if available;

- Please ensure that the RGA number is clearly written on the outside of the shipping package.

Once we receive the component and the RGA, an evaluation will be made to determine whether the component is defective and subject to warranty, or to estimate the cost of repair. One of our helpful technical support specialists will promptly contact you to discuss the results of our evaluation.

A purchase order will be required and an invoice will be issued should you wish to have a replacement part shipped prior to the completion of the warranty evaluation or repair. All invoices will be promptly offset with credit memos once we have received your part and determined it to be defective and subject to warranty replacement.

Douglas Machine shall be responsible for the repair or replacement of components that are, in its sole determination, defective and covered by warranty. This responsibility includes handling charges, non-expedited shipping charges to return a component to the Buyer, and all applicable duties, taxes and inspection fees.